Stepper Motor & Driver Setup Reference

HLTNC 57HS76-3004A08 (NEMA 23) + HLTNC TB6600 Driver — July 2026

# 1. Motor Details

| Parameter | Value |
| --- | --- |
| Model | HLTNC 57HS76-3004A08-D21-22 |
| Frame | NEMA 23 (57 mm), 76 mm body length |
| Type | Bipolar, 4-wire |
| Rated current | 3.0 A per phase |
| Driver used | HLTNC TB6600 (DC 9–42 V) |

# 2. Motor Wiring (per label on motor)

| Wire Colour | Coil / Polarity | TB6600 Terminal |
| --- | --- | --- |
| Red | Coil A + | A+ |
| Green | Coil A − | A− |
| Yellow | Coil B + | B+ |
| Blue | Coil B − | B− |

- Motor runs backwards? Swap Red and Green (or invert direction in software).

- Motor buzzes/jitters without turning? A wire is on the wrong coil — recheck pairs.

# 3. TB6600 Signal & Power Terminals

| Terminal | Function |
| --- | --- |
| PUL+ / PUL− | Step pulse input |
| DIR+ / DIR− | Direction input |
| ENA+ / ENA− | Enable (optional — leave unconnected to keep driver enabled) |
| VCC / GND | Motor power supply, DC 9–42 V |
| A+ A− B+ B− | Motor coils (see wiring table above) |

# 4. DIP Switch Settings (FINAL)

Important: on this driver, DOWN = ON, UP = OFF (marked “ON↓” on the switch block). Change switches only with power off.

| Switch | Setting | Lever Position |
| --- | --- | --- |
| SW1 | OFF | Up |
| SW2 | ON | Down |
| SW3 | OFF | Up |
| SW4 | OFF | Up |
| SW5 | OFF | Up |
| SW6 | ON | Down |

This gives:

- SW1–SW3 = OFF / ON / OFF → 1/8 microstepping, 1600 pulses per revolution.

- SW4–SW6 = OFF / OFF / ON → 2.8 A phase current (recommended for a 3.0 A motor on a TB6600 — cooler, near-full torque).

## Optional: full 3.0 A setting

If more torque is needed and the driver has good cooling: SW4 = OFF (up), SW5 = ON (down), SW6 = OFF (up). Expect more heat — heatsink/airflow required.

# 5. Notes

- Always verify against the table printed on the driver case — clone units can differ.

- Set current with power off; never disconnect motor wires while powered.

- At 1600 pulses/rev, one full revolution = 1600 step pulses from the controller.

- Give the TB6600 airflow at 2.8–3.0 A — it can misbehave when overheated.